# Name_____________________________________________

Date:_____________________________

AGSC 4353 – BUILDING REQUIREMENTS

## Calculating Concrete Problems

OBJECTIVE – To be able to determine the amount of ingredients needed in concrete slabs.

DIRECTIONS:  Calculate each problem.  To receive full credit for the correct answer, formulas and processes must be shown.   You may use your notes.

Using the Ready-mix method, determine the amount of concrete needed and the total costs for each.

1. Determine the cubic yards of concrete needed for a slab measuring 8’ X 200’ X 4” thick. Cu. Yds $

- Determine the cubic yards of concrete needed for a slab measuring 60’ x 200’ x 4”thick. Cu Yds. $

- Determine the cubic yards of concrete needed for a slab measuring 40’ x 80’ x 4” thick with a beam 12” wide and 12” deep at the perimeter of the slab. Cu. Yds. $

4. Using a 1 – 2 – 3 mix, calculate the amount of each ingredient needed for a sidewalk measuring10’ X 100’ X 4” thick. Use a 6-gallon mix. Use the Fuller Rule Method in the determining the ingredients needed. Refer to reference sheet for information. Show work on next page. 1. Cubic feet of concrete needed________________ - Cubic feet of raw materials _________________ - Total Mix needed__________________________ - Cubic yds of sand ________________________ - Cubic yds of gravel________________________ - Sacks of cement__________________________ - Amount of water needed ___________________ - Lbs of cement needed_____________________ - Tons of sand needed______________________ - Tons of gravel needed_____________________ - Total cost of sand ________________________ - Total cost of gravel _______________________ - Total cost of cement ______________________ - Total cost _______________________________

Refer to Problem 4: - How many bags of Easy Mix concrete mix are needed? __________________ - What is the total cost for this slab using the Easy Mix concrete mix? _______________ - Will a Utility trailer weighing 1500 pounds empty, rated at 7500 lbs. carrying capacity, how many trip will you have to make hauling Portland cement to the job site? ________________ - Using the same trailer in Problem 3, how many trips will be required to haul Portland cement to construction site? ______________________________ - Refer to Problem 4, how many cubic yard of ready-mix concrete is needed?_____________ - Which method is cheaper in materials? _______________________________________

Reference Table:

Following are actual costs of ingredients for concrete. Courtesy of Wright Materials, Inc, Home Depot and Alamo Cement. Sand = $9.05 per ton (1 cu yd –weighs 2700 lbs) Gravel = $15.05 per ton(1cu yd – weighs 2800 lbs) Portland Cement = $9.25 per bag (1 bag weighs 94 lbs) Easy Mix Concrete Mix = $3.90 per bag (1 bag weighs 80 lbs. and make .5 cubic feet of concrete Ready Mix Concrete = $85.50 per cubic yard

Show Work