Texas FFA Agricultural Mechanics Career Development Event

Area X Competition

Hosted by Texas A&M University - Kingsville

Sponsors:  Agricultural Consortium of Texas

Texas Agricultural Mechanics Committee

April 13, 2005

Superintendent – Jerome Tymrak

Theme for 2005 - "Integrated Pest Management"

Instructions: 	Congratulations on competing in the Area X CDE.  The written exam portion of the Agricultural Mechanics Career Development Event consists of 100 multiple-choice questions. Each multiple-choice question is worth one point.  You have 60 minutes to complete this portion of the CDE.  Make sure you are answering the questions in the proper place on the scantron and see that you are marking completely.  If you erase, please erase clearly and don't tear the scantron sheet.  PLEASE, DO NOT WRITE ON THE EXAM!  Thank you.  Good luck.

1.	When necessary to transport a boom sprayer on a public road or highway, always:

- use all required warning devices

- use the SMV emblem

- use warning lights

- all of the above.

IMS #8790, Understanding & Applying Safe Work Practices to Agriculture

2.	Recommended servicing of a boom sprayer includes:

- pre-season inspection

- preventive maintenance during spraying season

- regular maintenance and repair at the end of the spraying season

- all of the above

IMS # , FOS Mowers & Sprayers

3.	Which type of electric wire is suitable for installation in locations where both heat and moisture are concerns?

- THHN

- THWN

- TRW

- TW

IMS #8602-B, Electric Wiring

4.	Check the charge in a storage battery by measuring the:

- cold cranking amps

- specific gravity of each cell

- voltage drop between cells

- voltage drop in each cell

IMS #8794-E, Maintaining DC Electrical Systems

5.	Which of the following is the correct four-stroke combustion cycle sequence?

- exhaust, power, intake, compression

- intake, compression, power, exhaust

- intake, power, compression, exhaust

- power, exhaust, compression, intake

Harrell, Agricultural Power & Machinery

6.	In a turbocharged engine, _____ drives the impeller.

- an electric motor

- exhaust gasses

- a fan belt

- hydraulics

Harrell, Agricultural Power & Machinery

7.	The measure of electrical pressure or potential is _____.

- amperes

- ohms

- volts

- watts

IMS #8602-A, Electrical Principles, Terminology, & Safety

8.	Empty pesticide containers should be:

- disposed of in an EPA-approved landfill

- stored in a locked area

- tripled-rinsed and drained

- all of the above

IMS #4320, FBM Chemical Application Management

9.	In addition to protecting the engine block from freezing, engine coolant also:

- increases the boiling point

- increases the freezing point

- reduces engine knock

- serves as an octane booster.

IMS #, FOS Fuels, Coolants, & Lubricants

10.	A section of 12/3 WG NM sheathed cable will contain:

- one black wire, one white wire, one bare wire

- one black wire, one white wire, one green wire, one bare wire

- one black wire, one white wire, one red wire, one bare wire

- two black wires one white wire, one bare wire

IMS #8602-B, Electric Wiring

11.	Before changing the engine coolant, the operator should:

- add diesel fuel to the cleaning system for cooling

- flush the system to clean out rust and other deposits

- heat the solution to the boiling point

- pressurize the system to 20 psi.

IMS #4517, FOS Fuels, Coolants, & Lubricants

12.	Worn spray nozzles can cause:

- over-application

- the potential for chemical waste, crop burn, and environmental risk

- spray patterns to become uneven and spray coverage to be poor.

- all of the above

IMS #4320, FBM Chemical Application Management

13.	To extinguish an oil-based fire on a self-propelled boom-type sprayer, use a _____.

- type A fire extinguisher

- type B fire extinguisher

- type C fire extinguisher

- type D fire extinguisher

IMS #8790, Understanding & Applying Safe Work Practices to Agriculture

14.	Safety alert symbols are used on boom-type sprayers to help identify hazards.  Which signal word is used to identify the most potentially hazardous condition?

- Caution!

- Danger!

- Look out!

- Warning!

IMS #8790, Understanding & Applying Safe Work Practices to Agriculture

15.	Exposed wiring installed on the inside of walls and ceilings in pesticide storage facilities is called:

- exposed wiring

- hard wiring

- open wiring

- surface wiring

IMS #8231-C, Installing Electrical Circuits & Equipment

16.	The bolt grade with the highest SAE standard tensile strength is:

- Grade US #1

- Grade ISO 7

- Grade 5 Extra

- Grade 8

IMS #8617, Selecting & Using Wood & Metal Fasteners

17.	Before starting a self-propelled boom sprayer, which fluid levels should be checked?

a	coolant

- fuel & oil

- hydraulic

- all of the above

IMS #4517, FOS Fuels, Coolants & Lubricants

18.	To change the direction of rotation of a three-phase motor, simply:

- remove one of the conductors

- switch one current carrying conductor and the ground wire

- switch any two current carrying conductors

- switch the capacitor leads

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

19.	There are _____ square feet in one acre.

- 100

- 5,280

- 43,560

- 10,000

IMS #8772, Surveying Land

20.	A pesticide storage site should provide for all except:

- freedom from water damage & runoff

- non-porous flooring

- security

- seed & feed storage

IMS #4320, FBM Chemical Application Management

21.	The Society of Automotive Engineers (SAE) designates the:

- classification of engine oils

- combustion range of hydrocarbons

- detergent index of engine oils

- viscosity ranges of engine oils and gear lubricants.

IMS #8794-C, Selecting Lubricants and Maintaining Lubrication Systems

22.	A tripped circuit breaker on a 1/8 hp electric motor may be reset by:

- changing the relay

- pushing the breaker switch all the way off and turning it on again

- turning the motor off and pushing the restart button on the motor

- waiting 15 minutes and restarting the motor

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

23.	For a turf equipment storage building with four doors opening to the outside, use ____ to control the overhead lights from any door.

- four four-way switches

- four single-pole switches

- three three-way switches and one single pole switch

- two four-way and two three-way switches

8602-B, Electric Wiring

24.	Assuming that all other factors are constant, if actual travel speed of a boom sprayer is slower than the speed at which the sprayer was calibrated, the application rate _____.

- is not affected by travel speed if the sprayer was properly calibrated

- is higher

- is lower

- remains the same.

IMS #4320, FBM Chemical Application Management

25.	Neutral wires are attached to screws that are _____ in color.

- green

- silver

- white

- yellow

IMS #8602-B, Electric Wiring

26.	A 3 hp gasoline engine used to power a small sump pump can probably be replaced with a:

- 1 hp electric motor

- 2 hp electric motor

- 3 hp electric motor

- 4 hp electric motor

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

27.	A 5 hp electric motor used to operate a pump should be installed on:

- a general purpose branch circuit

- a general purpose feeder circuit

- an individual branch circuit

- a series circuit

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

28.	When should ANSI approved eye protection be worn?

- when using a grinder

- when using a hammer

- when using a wrench

- any time you are working in agricultural mechanics

IMS #8790, Understanding & Applying Safe Work Practices to Agriculture

29.	One mile per hour is equal to _____ feet per minute.

- 88

- 640

- 1620

- 5280

IMS #8792-A, Identifying & Selecting Machines and Equipment

30.	You inherit a Jinxco 15’ self-propelled boom sprayer valued at $3,000.  The sprayer needs approximately $375 worth of repairs and maintenance to get ready for use, and has an annual maintenance cost of $115.  Your municipality will pay $5 per acre to have recreational fields sprayed periodically.  How many acres must you spray over three years to pay for the repairs and maintenance costs?

- 144

- 444

- 744

- cannot be calculated from information given.

IMS #8792-A, Identifying & Selecting Machines and Equipment

31.	\Which of the following colors can be used for current-carrying conductors in an electrical circuit?

- blue

- black and red

- black, red, and blue

- conductor color does not matter

IMS #8602-B, Electric Wiring

32.	Because a motor may use more current when starting than when running, it is wise to:

- always use 14 gauge wire

- place overload protection on the neutral wire

- use at least a 30 amp buss fuse

- use overload protection that allows for a temporary overload without tripping

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

33.	Rate of flow in an electrical circuit is measured in _____.

- amperes

- ohms

- volts

- watts

IMS #8602-A, Electrical Principles, Terminology, & Safety

34.	A motor used to run a portable sump pump should be protected with a _____.

- buss fuse

- double trip breaker

- GFCI

- magnetic relay

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

35.	A dual-voltage electric motor operating a sump pump in a water hazard draws 13 amps at 120 volts and 6.5 amps at 240 volts.  The cost of operation would be:

- double if the motor is operated on 120 volts

- double if the motor is operated on 240 volts

- the same regardless of voltage used

- $0.75 per kwh

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

36.	The probability of a turf chemical being classified lethal depends on the LD50 rating.  The LD50 listed below with the highest toxicity rating is:

- 50

- 570

- 5000

- 7500

IMS #4320, FBM Chemical Application Management

37.	An additive that prevents particles of air from forming in the oil is called:

- anti-foam

- anti-oxidant

- hydrocarbon

- surfactant

IMS #8794-C, Selecting Lubricants and Maintaining Lubrication Systems

38.	When the start button is pushed on a motor starter that has a coil as part of its mechanism:

- the circuit is opened and the motor starts

- the coil is de-energized and the motor begins to run

- the coil is energized and the contacts close, providing energy for the motor to run

- the coil is energized and the contacts open, providing energy for the motor to run

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

39.	The tool used to test the protection level of an antifreeze and water solution is a:

- hydrometer

- near infrared sensor

- photo cell

- photo laser

IMS #4517, FOS Fuels, Coolants, & Lubricants

40.	The shape of the orifice on the spray nozzle determines _____.

- the angle of the s[ray pattern

- length and width of the spray pattern

- the shape of the spray pattern

- all of the above

IMS #4320, FBM Chemical Application Management

41.	In three-way circuits, electricity travels from one switch to another through _____.

- common terminal wires

- neutral wires

- traveler wires

- none of the above.

IMS #8602-B, Electric Wiring

42.	The formula for theoretical field capacity is Speed X Width / 8.25.  IF a self-propelled boom sprayer is moving at 3.25 mph and covering a width of 29 feet, the theoretical field capacity is _____.

- 6.5 acres/hour

- 6.86 acres/hour

- 7.87 acres/hour

- 7.95 acres/hour

IMS #8792, Adjusting, Calibrating, Maintaining and Operating Equipment

43.	A 1 hp motor that draws 16 amperes on 120 volts would draw _____ amperes on 240 volts.

- 8

- 16

- 32

- 40

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

44.	An electric motor generally uses about _____ watts to produce 1 horsepower of work.

- 15/25

- 288/420

- 746

- 1000

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

45.	Electric motors turning at full speed are rated in _____>

- cubic centimeters

- horsepower

- rpm

- watts

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

46.	A milliampere is _____.

- 1/10 of an ampere

- 1/100 of an ampere

- 1/1000 of an ampere

- 1/1,000,000 of an ampere

IMS #8602-A, Electrical Principles, Terminology, & Safety

47.	When reviewing MSDS data for pesticides, LD50 data is found in the _____.

- exposure control data

- health hazard data

- leak control data

- reactivity data

IMS #4320, FBM Chemical Application Management

48.	At engine operating temperature, the lubrication system temperature generally ranges:

- 150 – 175 degrees F

- 175 – 200 degrees F

- 200 – 225 degrees F

- 225 – 250 degrees F

IMS #8794-C, Selecting Lubricants and Maintaining Lubrication Systems

49.	When using portable motors of 1/3 hp or less, the motor controller can be _____.

- an attachment plug and receptacle

- a double pole switch

- a magnetic relay

- a single pole switch

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

50.	When filling a radiator with coolant, the proper level is _____.

- filled to ½ capacity

- filled to ¾ capacity

- full to the top of the radiator

- leave enough room for the coolant to expand.

IMS #4517, FOS Fuels, Coolants, & Lubricants

51.	When  boom sprayers, it is recommended to replace nozzles that are _____ above or below the desired application rate.

- 1%

- 5%

- 10%

- 12%

IMS #4320, FBM Chemical Application Management

52.	Electrical conductors are labeled to indicate gauge according to _____.

- American Welding Society sizes

- American Wire Gauge standards

- National Electrical Code

- United States Department of Weights and Measures

IMS #8602-A, Electrical Principles, Terminology, & Safety

53.	Which type of motor is not recommended for hard to start loads?

- capacitor-start

- split-phase

- thermal protected

- three-phase

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

54.	A control device on an electric motor must disconnect _____.

- all conductors

- all current-carrying conductors

- at least one current-carrying conductor

- the neutral conductor

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

55.	Electric motors may draw up to _____ times their full-load amperage when starting.

- 2

- 5

- 6

- 10

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

56.	Automatic control devices have _____.

- an overcurrent protection device

- a sensing element

- a switch

- a sensing element and a switch

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

57.	Electricity can travel back to it source through _____.

- another hot or neutral conductor

- earth

- both paths

IMS #8602-A, Electrical Principles, Terminology, & Safety

58.	A boom sprayer that travels 500 feet in 48 seconds is traveling _____ MPH.

- 5.2

- 7.1

- 8.3

- 10

IMS #8201-A, Safe Operation of Agricultural Tractors and Equipment

59.	The ignition quality of a diesel fuel is its _____ rating.

- cetane

- hydrocarbon

- octane

- saturated hydrocarbon

IMS #8794-D, Fuel System Maintenance: Fuel Selection, Storage, and Handling

60.	To operate an electric motor using a magnetic motor starter from two locations, use:

- two magnetic motor starters

- two magnetic motor starters with stop-start controls

- two start-stop stations with the start circuits wired in parallel

- two start-stop stations with the start circuits wired in series

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

61.	In a power shift transmission, _____.

- constant mesh gears are used so the transmission is continually engaged

- hydraulics power is used to shift the transmission and provide changes in speed and torque

- shift levers are used to change speed and direction of equipment

- sliding gears are used because they do not produce side thrust

Harrell, Agricultural Power & Machinery

62.	Application equipment that applies dry chemicals in dust or granular form uses a positive-type metering system to dispense the materials based on _____.

- direct injection

- particle size

- volume

- weight

IMS #4320, FBM Chemical Application Management

63.	A motor operating at 1750 rpm with a 3” pulley will turn a pump with a 2” pulley at _____ rpm.

- 820

- 1240

- 2630

- 3500

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

64.	An electric motor that operates a sump pump in a water hazard should be:

- dirt-proof

- explosion-proof

- moisture-proof

- splash-proof

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

65.	To connect an electric motor to the power source on a 120-volt, 3-wire power source, connect the green wire on the motor to the _____.

- bare wire of the cable

- brass screw on the motor

- green screw on the motor enclosure

- silver screw on the motor

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

66.	The duty rating on a motor nameplate means _____.

- life expectancy of the motor in years

- percent of time a motor can be run without overheating

- power produced by a motor

- whether the motor is water-proof.

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

67.	Variable rate technology uses three basic components.  Which is not one of those three components?

- actuators

- controllers

- rotators

- sensors

IMS #4598, FP The Precision Farming Guide for Agriculturists

68.	A variable resistor used to control voltage is a _____>

- capacitor

- interocitor

- potentiometer

- relay

IMS #8792-D, Identifying and Servicing Monitoring, Sensing, and Metering Devices

69.	Which of the following is not considered appropriate PPE for pesticide application?

- aprons & headgear

- eye protection

- leather gloves

- respirator

IMS #8792-E, Adjusting, Calibrating, Maintaining, and Operating Equipment

70.	A 5W-30 oil has a viscosity of 5 at _____.

- 212 degrees F

- 32 degrees F

- 10 degrees F

- –32 degrees F

IMS #8794-C, Selecting Lubricants and Maintaining Lubrication Systems

71.	Using the formula Speed X Width / 8.25, a 12’ wide self propelled boom sprayer moving at 3.5 mph can theoretically spray _____ acres per hour.

- 3.75

- 5.09

- 6.82

- 9.03

IMS #8792-E, Adjusting, Calibrating, Maintaining, and Operating Equipment

72.	Speedometers and tachometers geared to wheels or other parts of the drive train do not accurately indicate ground speed because they do not compensate for _____.

- tire inflation

- tire size

- tire slippage

- all of the above

IMS #4320, FBM Chemical Application Management

73	The formula for calculating horsepower is W (lbs) X D (ft.) X T (sec) divided by _____.

- 10

- 50

- 500

- 550

IMS #8792-E, Adjusting, Calibrating, Maintaining, and Operating Equipment

74.	Moving the right arm across the neck in a throat-cutting motion is an ASAE hand signal that means _____.

- raise the implement approximately 5’

- start the engine

- stop the engine

- stop forward travel

IMS #8790, Understanding & Applying Safe Work Practices

75.	Relays are incorporated into electric motor control circuits to _____.

- allow the use of low-amperage current in the control circuit

- protect motors from overheating due to overloading

- serve as step-up transformers to prevent voltage drop

- temporarily reduce starting voltage load

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

76.	Operator’s manuals recommend that fuel tanks be filled at the day’s end to _____.

- allow trash to settle out overnight

- decrease moisture condensation in the tank

- prevent trash from entering the tank

- reduce the need for a fuel filter

IMS #8794-D, Fuel System Maintenance, Fuel Selection, Storage, and Handling

77.	The amount of electric power produced is measured in _____.

- amperes

- ohms

- volts

- watts

IMS #8602-A, Electrical Principles, Terminology, & Safety

78.	With a power cost of $0.03 per kwh, it will cost approximately _____ to use a 5 hp electric motor to drain a water hazard containing approximately 35,000 gallons of water at a rate of 200 gallons per hour.

- $2.60

- $5.20

- $7.50

- $10.00

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

79.	The motor used in problem #78 is probably a _____.

- shaded-pole

- single-phase

- three-phase

- universal

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

80.	In Ohm’s Law, the “I” represents _____.

- amperage

- resistance

- voltage

- wattage

IMS #8602-A, Electrical Principles, Terminology, & Safety

81.	Material Safety Data Sheets (MSDS) provide pesticide-specific details on all factors except _____.

- emergency response techniques

- mixing instructions

- physical & health hazards

- safety procedures

IMS #4320, FBM Chemical Application Management

82.	Contamination of lubricants can be a serious problem in equipment maintenance.  The process that occurs when hydrocarbons in the oil combine with oxygen in the air to produce organic acids is called _____.

- aliphatization

- carbonization

- corrosion

- oxidation

IMS #8794-C, Selecting Lubricants and Maintaining Lubrication Systems

83.	A GFCI used with outdoor extension cords functions by _____.

- allowing a 50% circuit overload for short periods

- breaking the circuit if a current difference exists between the hot and ground wires

- breaking the circuit if a voltage difference exists between the hot and ground wires

- fusing the white or grounded wire to break the circuit

IMS #8602-A, Electrical Principles, Terminology, & Safety

84.	When spray system pressure is increased, droplet size at the nozzle orifice _____ as the rate of orifice wear increases.

- decreases

- increases

- increases proportionally with the amount of chemical in the mixture.

- remains the same

IMS #4320, FBM Chemical Application Management

85.	Three-way switch circuits for controlling lights usually include _____.

- grounded boxes

- three-wire cable

- two-wire cable

- all of the above

IMS #8602-B, Electric Wiring

86.	A drive sprocket with hooked teeth indicates a sprocket that _____.

- should be replaced

- uses double-pitch chains

- uses pintle-type chains

- uses single-pitch chains

IMS #4518, FOS Belts & Chains

87.	Use a _____ switch to control a three-phase motor.

- double-pole

- mercury

- single-pole

- three-pole

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

88.	Voltage drop is affected by _____.

- height of support

- length of conductor

- length and size of conductor

- size of conductor

IMS #8602-B, Electric Wiring

89.	Temperature rise as stated on a motor nameplate is the _____.

- ambient temperature for safe motor operation

- amount of heat the bearings are able to tolerate

- amount of heat the motor can generate during operation without causing damage

- none of the above

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

90.	A boom sprayer that travels 400 feet in 54 seconds is traveling _____ MPH.

- 4

- 5

- 6

- 7

IMS #8201-A, Safe Operation of Agricultural Tractors and Equipment

91.	Branch circuit conductors for motors must be rated for an amperage level not less than _____ of the motor’s full load rating.

- 75

- 90

- 100

- 125

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

92.	You plan to apply 3 pints of pesticide per acre in 11 gallons of spray.  How much pesticide must be added to a 300 gallon tank?

- 10.22 gallons

- 11.25 gallons

- 30.35 gallons

- 40.50 gallons

IMS #4320, FBM Chemical Application Management

93.	A quarter-section of land contains _____ acres.

- 160

- 640

- 1000

- 5280

IMS #8772, Surveying Land

94.	A rule for determining ballast requirements for mounted boom sprayers is _____.

- 1/3 of weight on front, 2/3 weight on rear

- ½ of weight on front, 1/2weight on rear

- 2/3 weight on front, 1/3 on rear

- ¾ weight on front, ¼ on rear

IMS #4600, FMO Tractors

95.	When calibrating chemical application equipment, the first step is to determine the _____ of the equipment during application.

- bulk tank capacity

- cost/benefit ratio

- field speed

- wind speed and direction

IMS #4320, FBM Chemical Application Management

96.	All electrical connections in circuits must be made in _____.

- boxes or fixtures

- by screws

- with solder

- with tape

IMS #8602-B, Electric Wiring

97.	The difference in NM and NMC cable is _____.

- NM is for inside use only

- NMC is for outside use only

- NMC is fungus and corrosion resistant

- None of the above

IMS #8231-C, Installing Electrical Circuits and Equipment

98.	The appropriate motor controller for a 240 v circuit is a _____.

- double pole

- single pole

- single pole, double throw

- triple pole

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

99.	A dual-voltage motor that draws 5 amps on 115 volts will draw _____ amps on 230 volts.

- 2.5

- 5

- 7.5

- 10

IMS #8795, Selecting, Operating, and Maintaining Electric Motors

100.	The first reading taken after moving and re-leveling is a _____.

- back sight

- bench mark

- fore sight

- turning point

IMS #8771, Selecting & Using Tripod Levels