Name: ____________________________________________

Date Completed: ___________________________________

AGSC 1352 – WELDING

SCORECARD

# ELECTRIC ARC WELDING ###### PIPE FILLET WELD #10 MATERIALS: - 1 piece of ¼”x 3”x 3” plate steel - 1 piece of 1-1/2” diameter x 2” black pipe - E6011 & or E7018 electrodes PROCEDURE: - Cut plate steel & pipe according to specification. - Place pipe in center of base metal - Use E6011 electrode for a stringer pass - Use E7018 electrode for a cover pass - Check for leaks by pouring water into pipe and let stand - DO NOT TURN IN HOT METAL ## PIPE FILLET WELD Scorecard Follow direction: 4 pts ________________ Bead width 5 pts _________________ Uniformity of bead 5 pts _________________ Bead appearance 10 pts _________________ Techniques used 5 pts ________________ Penetration 7 pts ________________ Water holding capabilities 10 pts ________________ Safety 3 pts ________________ Total 50 pts ________________

Texas A&M University-Kingsville – Ag Mechanics Dept

Name: ____________________________________________

Date Completed: ___________________________________

AGSC 1451

SCORECARD

# Oxy-Acetylene Cutting MATERIALS: - 1 piece – ¼” x 5” steel plate PROCEDURE: - Cut steel bar according to specification, illustrated below. - Measure & mark layout - Use a try square & soapstone to square end. - Use proper procedures in setup & operating OA cutting equipment. - Clean tip orifices. - Use chart to determine pressures for cutting operation. - Cut marked area. - Cut according to specifications. DO NOT TURN IN HOT 5” 1-1/2” OXY-ACETYLENE CUTTING SKILL Scorecard Follow direction: 3 pts _________________ Measurements 20pts __________________ Uniform cuts 10 pts __________________ Visible slag 5 pts _________________ Evidence of beveling 10 pts ________________ Safety 2 pts ________________ Total 50 pts ________________

Texas A&M University-Kingsville – Ag Mechanics Dept