Name_________________________________________

Date completed________________________________

AGSC 1451 – CARPENTRY (Folding Stool)

Objectives:

- Be able to use hand tools & power equipment properly and safely.

- Be able to use a variety of tools and power equipment to construct a wood project.

- Be able to recognize quality workmanship through judging skills & techniques.

Bills of Materials

- 1 piece of lumber ¾” X 10” X 10’

- 1 – 36inch wooden dowel rod

- 24 – 1-1/2” # 6 drywall screws

- 4 – 1” brad nails

Directions:

Step 1:  Use of Table Saw

- Rip lumber into 1-3/4 inch wide

Step 2:  Use of Radial Arm Saw

- Cut 4 pieces 22 inches in length (legs)

- Cut 4 pieces 14 inches in length (seat support

- Cut 8 pieces 17 inches in length (seat)

- Save remaining pieces for braces-cut later

Step 3:  Use of Planer

- Plane pieces cut above to measure 1-1/2” in width.

Step 4:  Use of Drill Press

- Drill holes as indicated below using a ¾” forstner bit  (legs)

1”            10”			   11”

- Drill holes as indicated below (seat supports)

1”

Step 5:  Use of Power Disc Sander

- Legs - round corners (see diagram below)

- Use an 1-1/2” flat washer to measure corners

1-1/2”

- Seat supports – round one corner

Step 6:  Use of Band Saw

- Cut 4 pieces of dowel rod 1-1/2” in length

- Cut 1 piece of dowel rod 12” in length

Step 7:  Use of power drills & screwdrivers

- See assembly below

- Use # 6 1-1/4”  drywall screws to secure the seat portions to the seat support.

- Countersink screws – screws must be flush with surface.

9”